NVIDIA Regional R&D Hub

Device Configuration Documentation

Overview

1. Spine-1

| Property | Value |
| --- | --- |
| Device type | Catalyst 3650 Layer 3 Switch |
| Role | Core fabric — OSPF routing, no VLANs, no end devices |
| Interfaces used | Gig1/0/1 → Leaf-1, Gig1/0/2 → Leaf-2, Gig1/0/3 → Leaf-3 |
| Unused ports | Gig1/0/4 through Gig1/0/24, Gig1/1/1 through Gig1/1/4 — all shut down |

Configuration

enable configure terminal ip routing interface gigabitEthernet1/0/1 no switchport ip address 10.0.1.1 255.255.255.252 no shutdown exit interface gigabitEthernet1/0/2 no switchport ip address 10.0.1.5 255.255.255.252 no shutdown exit interface gigabitEthernet1/0/3 no switchport ip address 10.0.1.9 255.255.255.252 no shutdown exit router ospf 1 network 10.0.1.0 0.0.0.3 area 0 network 10.0.1.4 0.0.0.3 area 0 network 10.0.1.8 0.0.0.3 area 0 exit interface range gigabitEthernet1/0/4 - 24 shutdown exit interface range gigabitEthernet1/1/1 - 4 shutdown exit end write memory

2. Spine-2

| Property | Value |
| --- | --- |
| Device type | Catalyst 3650 Layer 3 Switch |
| Role | Core fabric — redundant path, ECMP with Spine-1 |
| Interfaces used | Gig1/0/1 → Leaf-1, Gig1/0/2 → Leaf-2, Gig1/0/3 → Leaf-3 |
| Unused ports | Gig1/0/4 through Gig1/0/24, Gig1/1/1 through Gig1/1/4 — all shut down |

Configuration

enable configure terminal ip routing interface gigabitEthernet1/0/1 no switchport ip address 10.0.2.1 255.255.255.252 no shutdown exit interface gigabitEthernet1/0/2 no switchport ip address 10.0.2.5 255.255.255.252 no shutdown exit interface gigabitEthernet1/0/3 no switchport ip address 10.0.2.9 255.255.255.252 no shutdown exit router ospf 1 network 10.0.2.0 0.0.0.3 area 0 network 10.0.2.4 0.0.0.3 area 0 network 10.0.2.8 0.0.0.3 area 0 exit interface range gigabitEthernet1/0/4 - 24 shutdown exit interface range gigabitEthernet1/1/1 - 4 shutdown exit end write memory

3. Leaf-1 (Compute)

| Property | Value |
| --- | --- |
| Device type | Catalyst 3650 Layer 3 Switch |
| Role | Department aggregation, inter-VLAN routing, DHCP relay, ACL enforcement, SSH, AAA, Guest VLAN |
| VLANs hosted | 10 (IT), 20 (Production), 30 (Management), 40 (Support Sector 1), 50 (Support Sector 2), 60 (Study), 99 (Guest) |
| Spine uplinks | Gig1/0/1 → Spine-1, Gig1/0/2 → Spine-2 |
| Trunk downlinks | Gig1/0/3-8 → 6 access switches (one per department) |
| Guest port | Gig1/0/9 → Guest Laptop (VLAN 99) |
| Unused ports | Gig1/0/10 through Gig1/0/24, Gig1/1/1 through Gig1/1/4 — all shut down |

3.1 VLAN Creation

enable configure terminal vlan 10 name IT exit vlan 20 name Production exit vlan 30 name Management exit vlan 40 name SupportSector1 exit vlan 50 name SupportSector2 exit vlan 60 name Study exit vlan 99 name Guest exit end write memory

3.2 Routed Uplinks to Spines

enable configure terminal ip routing interface gigabitEthernet1/0/1 no switchport ip address 10.0.1.2 255.255.255.252 no shutdown exit interface gigabitEthernet1/0/2 no switchport ip address 10.0.2.2 255.255.255.252 no shutdown exit end write memory

3.3 Trunk Downlinks to Access Switches

enable configure terminal interface gigabitEthernet1/0/3 switchport mode trunk switchport trunk allowed vlan 10 exit interface gigabitEthernet1/0/4 switchport mode trunk switchport trunk allowed vlan 20 exit interface gigabitEthernet1/0/5 switchport mode trunk switchport trunk allowed vlan 30 exit interface gigabitEthernet1/0/6 switchport mode trunk switchport trunk allowed vlan 40 exit interface gigabitEthernet1/0/7 switchport mode trunk switchport trunk allowed vlan 50 exit interface gigabitEthernet1/0/8 switchport mode trunk switchport trunk allowed vlan 60 exit end write memory

3.4 Guest Port

enable configure terminal interface gigabitEthernet1/0/9 switchport switchport mode access switchport access vlan 99 spanning-tree portfast no shutdown exit end write memory

3.5 SVIs (Department Gateways + DHCP Relay)

enable configure terminal interface vlan 10 ip address 192.168.10.254 255.255.255.0 ip helper-address 192.168.70.11 no shutdown exit interface vlan 20 ip address 192.168.20.254 255.255.255.0 ip helper-address 192.168.70.11 no shutdown exit interface vlan 30 ip address 192.168.30.254 255.255.255.0 ip helper-address 192.168.70.11 no shutdown exit interface vlan 40 ip address 192.168.40.254 255.255.255.0 ip helper-address 192.168.70.11 no shutdown exit interface vlan 50 ip address 192.168.50.254 255.255.255.0 ip helper-address 192.168.70.11 no shutdown exit interface vlan 60 ip address 192.168.60.254 255.255.255.0 ip helper-address 192.168.70.11 no shutdown exit interface vlan 99 ip address 192.168.99.254 255.255.255.0 ip helper-address 192.168.70.11 no shutdown exit end write memory

3.6 OSPF

enable configure terminal router ospf 1 passive-interface default no passive-interface gigabitEthernet1/0/1 no passive-interface gigabitEthernet1/0/2 network 10.0.1.0 0.0.0.3 area 0 network 10.0.2.0 0.0.0.3 area 0 network 192.168.10.0 0.0.0.255 area 0 network 192.168.20.0 0.0.0.255 area 0 network 192.168.30.0 0.0.0.255 area 0 network 192.168.40.0 0.0.0.255 area 0 network 192.168.50.0 0.0.0.255 area 0 network 192.168.60.0 0.0.0.255 area 0 network 192.168.99.0 0.0.0.255 area 0 exit end write memory

3.7 SSH Configuration

enable configure terminal ip domain-name nvidia.local username admin privilege 15 secret Cisco123 username backup_admin secret Cisco123 crypto key generate rsa 1024 ip ssh version 2 line vty 0 15 transport input ssh login local exit end write memory

3.8 AAA Framework

radius-server host command is not supported by PT Catalyst 3650 simulation — documented as PT limitation.

enable configure terminal aaa new-model aaa authentication login default group radius local end write memory

3.9 Unused Port Shutdown

enable configure terminal interface range gigabitEthernet1/0/10 - 24 shutdown exit interface range gigabitEthernet1/1/1 - 4 shutdown exit end write memory

4. Leaf-2 (Services)

| Property | Value |
| --- | --- |
| Device type | Catalyst 3650 Layer 3 Switch |
| Role | Internal server aggregation, VLAN 70 and VLAN 90 gateway |
| VLANs hosted | 70 (Servers), 90 (iSCSI) |
| Spine uplinks | Gig1/0/1 → Spine-1, Gig1/0/2 → Spine-2 |
| Server ports | Gig1/0/3 DNS, Gig1/0/4 DHCP, Gig1/0/5 AAA, Gig1/0/6 iSCSI (VLAN 90), Gig1/0/7 Compute, Gig1/0/8 Internal-FTP, Gig1/0/9 Syslog |
| Unused ports | Gig1/0/10 through Gig1/0/24, Gig1/1/1 through Gig1/1/4 — all shut down |

Configuration

enable configure terminal vlan 70 name Servers exit vlan 90 name iSCSI exit ip routing interface gigabitEthernet1/0/1 no switchport ip address 10.0.1.6 255.255.255.252 no shutdown exit interface gigabitEthernet1/0/2 no switchport ip address 10.0.2.6 255.255.255.252 no shutdown exit interface gigabitEthernet1/0/3 switchport mode access switchport access vlan 70 spanning-tree portfast no shutdown exit interface gigabitEthernet1/0/4 switchport mode access switchport access vlan 70 spanning-tree portfast no shutdown exit interface gigabitEthernet1/0/5 switchport mode access switchport access vlan 70 spanning-tree portfast no shutdown exit interface gigabitEthernet1/0/6 switchport mode access switchport access vlan 90 spanning-tree portfast no shutdown exit interface gigabitEthernet1/0/7 switchport mode access switchport access vlan 70 spanning-tree portfast no shutdown exit interface gigabitEthernet1/0/8 switchport mode access switchport access vlan 70 spanning-tree portfast no shutdown exit interface gigabitEthernet1/0/9 switchport mode access switchport access vlan 70 spanning-tree portfast no shutdown exit interface vlan 70 ip address 192.168.70.254 255.255.255.0 no shutdown exit interface vlan 90 ip address 192.168.90.254 255.255.255.0 no shutdown exit router ospf 1 passive-interface default no passive-interface gigabitEthernet1/0/1 no passive-interface gigabitEthernet1/0/2 network 10.0.1.4 0.0.0.3 area 0 network 10.0.2.4 0.0.0.3 area 0 network 192.168.70.0 0.0.0.255 area 0 network 192.168.90.0 0.0.0.255 area 0 exit interface range gigabitEthernet1/0/10 - 24 shutdown exit interface range gigabitEthernet1/1/1 - 4 shutdown exit ip domain-name nvidia.local username admin privilege 15 secret Cisco123 crypto key generate rsa 1024 ip ssh version 2 line vty 0 15 transport input ssh login local exit end write memory

5. Leaf-3 (Edge)

| Property | Value |
| --- | --- |
| Device type | Catalyst 3650 Layer 3 Switch |
| Role | Edge transit, DMZ gateway, default route origination via OSPF |
| VLANs hosted | 80 (DMZ) |
| Spine uplinks | Gig1/0/1 → Spine-1, Gig1/0/2 → Spine-2 |
| Firewall link | Gig1/0/3 → ASA Firewall Gig1/2 (inside) |
| DMZ ports | Gig1/0/4 → DMZ-Web, Gig1/0/5 → DMZ-FTP |
| Unused ports | Gig1/0/6 through Gig1/0/24, Gig1/1/1 through Gig1/1/4 — all shut down |

Configuration

enable configure terminal vlan 80 name DMZ exit ip routing interface gigabitEthernet1/0/1 no switchport ip address 10.0.1.10 255.255.255.252 no shutdown exit interface gigabitEthernet1/0/2 no switchport ip address 10.0.2.10 255.255.255.252 no shutdown exit interface gigabitEthernet1/0/3 no switchport ip address 10.0.3.1 255.255.255.252 no shutdown exit interface gigabitEthernet1/0/4 switchport mode access switchport access vlan 80 spanning-tree portfast no shutdown exit interface gigabitEthernet1/0/5 switchport mode access switchport access vlan 80 spanning-tree portfast no shutdown exit interface vlan 80 ip address 192.168.80.254 255.255.255.0 no shutdown exit ip route 0.0.0.0 0.0.0.0 10.0.3.2 router ospf 1 passive-interface default no passive-interface gigabitEthernet1/0/1 no passive-interface gigabitEthernet1/0/2 default-information originate network 10.0.1.8 0.0.0.3 area 0 network 10.0.2.8 0.0.0.3 area 0 network 192.168.80.0 0.0.0.255 area 0 exit interface range gigabitEthernet1/0/6 - 24 shutdown exit interface range gigabitEthernet1/1/1 - 4 shutdown exit end write memory

6. Access Switches

All 6 access switches follow the same configuration pattern. Only the hostname, VLAN number, VLAN name, and port range differ between them. Each switch carries a single department VLAN only.

6.1 IT-Access-Switch

enable configure terminal hostname IT-Access-Switch vlan 10 name IT exit interface gigabitEthernet0/1 switchport mode trunk switchport trunk allowed vlan 10 exit interface range fastEthernet0/1 - 5 switchport mode access switchport access vlan 10 spanning-tree portfast exit interface range fastEthernet0/6 - 24 shutdown exit aaa new-model aaa authentication dot1x default group radius dot1x system-auth-control username admin privilege 15 secret Cisco123 end write memory

6.2 Prod-Access-Switch

enable configure terminal hostname Prod-Access-Switch vlan 20 name Production exit interface gigabitEthernet0/1 switchport mode trunk switchport trunk allowed vlan 20 exit interface range fastEthernet0/1 - 10 switchport mode access switchport access vlan 20 spanning-tree portfast exit interface range fastEthernet0/11 - 24 shutdown exit end write memory

6.3 Mgmt-Access-Switch

enable configure terminal hostname Mgmt-Access-Switch vlan 30 name Management exit interface gigabitEthernet0/1 switchport mode trunk switchport trunk allowed vlan 30 exit interface range fastEthernet0/1 - 6 switchport mode access switchport access vlan 30 spanning-tree portfast exit interface range fastEthernet0/7 - 24 shutdown exit end write memory

6.4 Support-Switch-1

enable configure terminal hostname Support-Switch-1 vlan 40 name SupportSector1 exit interface gigabitEthernet0/1 switchport mode trunk switchport trunk allowed vlan 40 exit interface range fastEthernet0/1 - 11 switchport mode access switchport access vlan 40 spanning-tree portfast exit interface range fastEthernet0/12 - 24 shutdown exit end write memory

6.5 Support-Switch-2

enable configure terminal hostname Support-Switch-2 vlan 50 name SupportSector2 exit interface gigabitEthernet0/1 switchport mode trunk switchport trunk allowed vlan 50 exit interface range fastEthernet0/1 - 11 switchport mode access switchport access vlan 50 spanning-tree portfast exit interface range fastEthernet0/12 - 24 shutdown exit end write memory

6.6 Study-Access-Switch

enable configure terminal hostname Study-Access-Switch vlan 60 name Study exit interface gigabitEthernet0/1 switchport mode trunk switchport trunk allowed vlan 60 exit interface range fastEthernet0/1 - 9 switchport mode access switchport access vlan 60 spanning-tree portfast exit interface range fastEthernet0/10 - 24 shutdown exit end write memory

7. Edge Router

| Property | Value |
| --- | --- |
| Device type | Cisco ISR Router |
| Role | Internet connectivity — connects Internet Cloud to ASA Firewall |
| Gig0/0 | 203.0.113.1 — internet-facing, connects to Internet Cloud |
| Gig0/1 | 203.0.113.5 — firewall-facing, connects to ASA Firewall Gig1/1 |
| Loopback0 | 8.8.8.8 — simulated internet destination for testing |
| Default route | Points out Gig0/0 toward Internet Cloud |
| ACL | EDGE-PROTECT — blocks direct access to router management IPs from internet |

Configuration

enable configure terminal interface gigabitEthernet0/0 ip address 203.0.113.1 255.255.255.252 no shutdown exit interface gigabitEthernet0/1 ip address 203.0.113.5 255.255.255.252 no shutdown exit interface loopback0 ip address 8.8.8.8 255.255.255.255 no shutdown exit ip route 0.0.0.0 0.0.0.0 gigabitEthernet0/0 ip access-list extended EDGE-PROTECT deny ip any host 203.0.113.1 deny ip any host 203.0.113.5 deny icmp any any echo permit ip any any exit interface gigabitEthernet0/0 ip access-group EDGE-PROTECT in exit logging 192.168.70.16 logging trap 6 end write memory

8. ASA Firewall

| Property | Value |
| --- | --- |
| Device type | Cisco ASA 5506 (ASA Version 9.6) |
| Role | Perimeter security, NAT/PAT, inbound access control, Syslog forwarding |
| Gig1/1 (outside) | 203.0.113.6, security-level 0 — faces Edge Router |
| Gig1/2 (inside) | 10.0.3.2, security-level 100 — faces Leaf-3 |
| NAT type | PAT (object-based syntax — required for ASA 9.6) |
| Outside ACL | OUTSIDE-IN — permits HTTP/HTTPS to DMZ-Web, FTP to DMZ-FTP, ICMP echo-reply |
| Static routes | Default route via Edge Router, return route for 192.168.0.0/16 via Leaf-3 |

ASA 9.6 requires object-based NAT syntax. Legacy nat/global commands were removed in ASA 8.3 and silently fail on this platform.

8.1 Interface Configuration

enable configure terminal interface gigabitEthernet1/1 nameif outside security-level 0 ip address 203.0.113.6 255.255.255.252 no shutdown exit interface gigabitEthernet1/2 nameif inside security-level 100 ip address 10.0.3.2 255.255.255.252 no shutdown exit end write memory

8.2 Static Routes

enable configure terminal route outside 0.0.0.0 0.0.0.0 203.0.113.5 route inside 192.168.0.0 255.255.0.0 10.0.3.1 end write memory

8.3 NAT/PAT (Object-Based)

enable configure terminal object network Internal-Networks subnet 0.0.0.0 0.0.0.0 nat (inside,outside) dynamic interface exit end write memory

8.4 Outside Interface ACL

enable configure terminal access-list OUTSIDE-IN extended permit tcp any host 192.168.80.10 eq 80 access-list OUTSIDE-IN extended permit tcp any host 192.168.80.10 eq 443 access-list OUTSIDE-IN extended permit tcp any host 192.168.80.11 eq 21 access-list OUTSIDE-IN extended permit tcp any host 192.168.80.11 eq 20 access-list OUTSIDE-IN extended permit icmp any any echo-reply access-group OUTSIDE-IN in interface outside end write memory

8.5 ICMP Inspection Policy

enable configure terminal policy-map global_policy class inspection_default inspect icmp exit service-policy global_policy global end write memory

8.6 Syslog

enable configure terminal logging enable logging host inside 192.168.70.16 logging trap 6 end write memory

9. Servers

Servers in Packet Tracer are configured via the Config tab (for IP addressing) and Services tab (for service enablement) — no CLI is available. All configuration below describes the settings applied through these graphical tabs.

9.1 Server IP Addressing (Config Tab)

| Server | IP Address | Subnet Mask | Gateway | Connected To |
| --- | --- | --- | --- | --- |
| DNS | 192.168.70.10 | 255.255.255.0 | 192.168.70.254 | Leaf-2 Gig1/0/3 |
| DHCP | 192.168.70.11 | 255.255.255.0 | 192.168.70.254 | Leaf-2 Gig1/0/4 |
| AAA/RADIUS | 192.168.70.12 | 255.255.255.0 | 192.168.70.254 | Leaf-2 Gig1/0/5 |
| iSCSI Storage | 192.168.90.10 | 255.255.255.0 | 192.168.90.254 | Leaf-2 Gig1/0/6 |
| Central-Compute | 192.168.70.14 | 255.255.255.0 | 192.168.70.254 | Leaf-2 Gig1/0/7 |
| Internal-FTP | 192.168.70.15 | 255.255.255.0 | 192.168.70.254 | Leaf-2 Gig1/0/8 |
| Syslog | 192.168.70.16 | 255.255.255.0 | 192.168.70.254 | Leaf-2 Gig1/0/9 |
| DMZ-Web | 192.168.80.10 | 255.255.255.0 | 192.168.80.254 | Leaf-3 Gig1/0/4 |
| DMZ-FTP | 192.168.80.11 | 255.255.255.0 | 192.168.80.254 | Leaf-3 Gig1/0/5 |

9.2 Services Enabled (Services Tab)

| Server | Service Configuration | Notes |
| --- | --- | --- |
| DNS | DNS On | All other services Off |
| DHCP | DHCP On — 7 pools configured | All other services Off |
| AAA/RADIUS | AAA On — test account nvidia_user created | All other services Off |
| iSCSI Storage | All services Off | No iSCSI service available in PT — represented by address only |
| Central-Compute | All services Off | No AI/LLM service available in PT — represented by address only |
| Internal-FTP | FTP On — shared_user account created with R/W/List | All other services Off |
| Syslog | SYSLOG On | All other services Off |
| DMZ-Web | HTTP On | All other services Off |
| DMZ-FTP | FTP On — account created with appropriate permissions | All other services Off |

9.3 DHCP Pools (DHCP Server Services Tab)

| Pool Name | Start IP | Default Gateway | DNS Server |
| --- | --- | --- | --- |
| IT | 192.168.10.20 | 192.168.10.254 | 192.168.70.10 |
| Production | 192.168.20.20 | 192.168.20.254 | 192.168.70.10 |
| Management | 192.168.30.20 | 192.168.30.254 | 192.168.70.10 |
| SupportSector1 | 192.168.40.20 | 192.168.40.254 | 192.168.70.10 |
| SupportSector2 | 192.168.50.20 | 192.168.50.254 | 192.168.70.10 |
| Study | 192.168.60.20 | 192.168.60.254 | 192.168.70.10 |
| Guest | 192.168.99.20 | 192.168.99.254 | 192.168.70.10 |

9.4 DNS Records (DNS Server Services Tab)

| Name | Type | Address | Purpose |
| --- | --- | --- | --- |
| www.nvidia.local | A Record | 192.168.80.10 | Company internal website |
| ftp.nvidia.local | A Record | 192.168.80.11 | Company FTP server |
| www.google.com | A Record | 192.168.80.10 | Simulated internet demo — points to DMZ-Web |

10. Packet Tracer Simulation Limitations

The following configurations were correctly applied but are not enforced or simulated by Packet Tracer. Each is documented here as a production-vs-simulation gap. The configurations themselves are correct for real Cisco hardware.

| Feature | Device | PT Behavior | Real Hardware Behavior |
| --- | --- | --- | --- |
| SVI ACL enforcement | Leaf-1 VLANs 10-60 | Department ACLs created and attached — PT does not enforce | Would enforce on real Catalyst 3650 |
| SVI ACL enforcement | Leaf-3 VLAN 80 | DMZ-ISOLATION ACL created and attached — PT does not enforce | Would enforce on real Catalyst 3650 |
| GUEST-ACL enforcement | Leaf-1 VLAN 99 | GUEST-ACL applied and confirmed enforcing in PT | Working correctly |
| radius-server host | Leaf-1, IT-Access-Switch | Command rejected as invalid input by PT | Accepted on real hardware |
| dot1x port-control auto | IT-Access-Switch | Applied but not persisted by PT | Persisted on real hardware |
| Syslog on switches | All Catalyst switches | logging command rejected by PT simulation | Accepted on real hardware |
| iSCSI protocol | iSCSI server | No iSCSI service available in PT Services tab | Full iSCSI stack on real hardware |
| AI/LLM workload | Central-Compute | No AI/LLM service available in PT | Actual compute workload on real hardware |
| SFTP | DMZ-FTP | No SFTP service available in PT — FTP used instead | SFTP available on real hardware |
| Dynamic ARP Inspection | All access switches | DHCP Snooping binding database not populated by PT | Fully functional on real hardware |
| StackWise stacking | All Leaf switches | Not simulatable in PT | Deployed as stacked pairs in production |