VLAN &SUBNET WORKSHEET

| VLAN ID | Subnet Name | Needed Hosts | Subnet Address | Gateway IP | IP Range | Subnet Mask | Usable IPs |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | IT Department | 5 | 192.168.10.0 | 192.168.10.254 | .1 - .253 | 255.255.255.0 | 253 |
| 20 | Production | 10 | 192.168.20.0 | 192.168.20.254 | .1 - .253 | 255.255.255.0 | 253 |
| 30 | Management | 5+1 | 192.168.30.0 | 192.168.30.254 | .1 - .253 | 255.255.255.0 | 253 |
| 40 | Support Sector 1 | 10+1 | 192.168.40.0 | 192.168.40.254 | .1 - .253 | 255.255.255.0 | 253 |
| 50 | Support Sector 2 | 10+1 | 192.168.50.0 | 192.168.50.254 | .1 - .253 | 255.255.255.0 | 253 |
| 60 | Study | 8+1 | 192.168.60.0 | 192.168.60.254 | .1 - .253 | 255.255.255.0 | 253 |
| 70 | Servers | 7 | 192.168.70.0 | 192.168.70.254 | .1 - .253 | 255.255.255.0 | 253 |
| 80 | DMZ | 2 | 192.168.80.0 | 192.168.80.254 | .1 - .253 | 255.255.255.0 | 253 |
| 90 | iSCSI | 1 | 192.168.90.0 | 192.168.90.254 | .1 - .253 | 255.255.255.0 | 253 |
| 99 | Guest | variable | 192.168.99.0 | 192.168.99.254 | .1 - .253 | 255.255.255.0 | 253 |

DHCP POOL CONFIGURATION

| VLAN ID | POOL NAME | Start IP | End IP | Gateway | DNS Server |
| --- | --- | --- | --- | --- | --- |
| 10 | IT Department | 192.168.10.20 | 192.168.10.253 | 192.168.10.254 | 192.168.70.10 |
| 20 | Production | 192.168.20.20 | 192.168.20.253 | 192.168.20.254 | 192.168.70.10 |
| 30 | Management | 192.168.30.20 | 192.168.30.253 | 192.168.30.254 | 192.168.70.10 |
| 40 | Support Sector 1 | 192.168.40.20 | 192.168.40.253 | 192.168.40.254 | 192.168.70.10 |
| 50 | Support Sector 2 | 192.168.50.20 | 192.168.50.253 | 192.168.50.254 | 192.168.70.10 |
| 60 | Study | 192.168.60.20 | 192.168.60.253 | 192.168.60.254 | 192.168.70.10 |
| 99 | Guest | 192.168.99.20 | 192.168.99.253 | 192.168.99.254 | 192.168.70.10 |

Note: Servers (VLAN 70), DMZ (VLAN 80), and iSCSI (VLAN 90) use static IPs only — no DHCP pools for these VLANs.

STATIC IP ASSIGNMENTS – SERVERS

| Device | VLAN | IP Address | Gateway | Service | Type |
| --- | --- | --- | --- | --- | --- |
| DNS Server | 70 | 192.168.70.10 | 192.168.70.254 | DNS | Mandatory |
| DHCP Server | 70 | 192.168.70.11 | 192.168.70.254 | DHCP | Mandatory |
| AAA/RADIUS Server | 70 | 192.168.70.12 | 192.168.70.254 | RADIUS | Bonus |
| iSCSI Storage | 90 | 192.168.90.10 | 192.168.90.254 | iSCSI | Bonus |
| Central-Compute-LLM | 70 | 192.168.70.14 | 192.168.70.254 | AI Compute | Mandatory |
| Internal-FTP | 70 | 192.168.70.15 | 192.168.70.254 | FTP | Team addition |
| Syslog | 70 | 192.168.70.16 | 192.168.70.254 | Syslog | Team addition |
| DMZ-Web | 80 | 192.168.80.10 | 192.168.80.254 | HTTP | Bonus |
| DMZ-FTP | 80 | 192.168.80.11 | 192.168.80.254 | FTP | Mandatory |

POINT TO POINT INFRASTRUCTURE LINKS

| Link | Subnet | Device A | IP A | Device B | IP B |
| --- | --- | --- | --- | --- | --- |
| Spine-1 ↔ Leaf-1 | 10.0.1.0/30 | Spine-1 Gig1/0/1 | 10.0.1.1 | Leaf-1 Gig1/0/1 | 10.0.1.2 |
| Spine-1 ↔ Leaf-2 | 10.0.1.4/30 | Spine-1 Gig1/0/2 | 10.0.1.5 | Leaf-2 Gig1/0/1 | 10.0.1.6 |
| Spine-1 ↔ Leaf-3 | 10.0.1.8/30 | Spine-1 Gig1/0/3 | 10.0.1.9 | Leaf-3 Gig1/0/1 | 10.0.1.10 |
| Spine-2 ↔ Leaf-1 | 10.0.2.0/30 | Spine-2 Gig1/0/1 | 10.0.2.1 | Leaf-1 Gig1/0/2 | 10.0.2.2 |
| Spine-2 ↔ Leaf-2 | 10.0.2.4/30 | Spine-2 Gig1/0/2 | 10.0.2.5 | Leaf-2 Gig1/0/2 | 10.0.2.6 |
| Spine-2 ↔ Leaf-3 | 10.0.2.8/30 | Spine-2 Gig1/0/3 | 10.0.2.9 | Leaf-3 Gig1/0/2 | 10.0.2.10 |
| Leaf-3 ↔ Firewall | 10.0.3.0/30 | Leaf-3 Gig1/0/3 | 10.0.3.1 | Firewall Gig1/2 | 10.0.3.2 |

EDGE/INTERNET ADDRESSING

| Device | Interface | IP Address | Subnet | Purpose |
| --- | --- | --- | --- | --- |
| Edge Router | Gig0/0 | 203.0.113.1 | /30 | Internet-facing |
| Edge Router | Gig0/1 | 203.0.113.5 | /30 | Firewall-facing |
| Edge Router | Loopback0 | 8.8.8.8 | /32 | Simulated internet target |
| ASA Firewall | Gig1/1 (outside) | 203.0.113.6 | /30 | Internet-facing |
| ASA Firewall | Gig1/2 (inside) | 10.0.3.2 | /30 | Internal-facing |

Note: 203.0.113.0/24 is IANA-reserved documentation range — used to represent public address space in this simulation.

Why /24 for every VLAN:

Every VLAN was assigned a full /24 rather than a tight VLSM block. This provides growth headroom, simplifies addressing (VLAN ID matches third octet), and avoids renumbering if department sizes change. No address scarcity exists in the 192.168.0.0/16 private range for this scale.

Why iSCSI has its own VLAN 90:

iSCSI was originally on VLAN 70 alongside other servers, with a planned host-level ACL restricting access to Production only. Moving it to a dedicated VLAN 90 provides stronger isolation — non-Production departments have no Layer 3 path to 192.168.90.0/24 at all, rather than relying solely on an ACL as the single enforcement point.

Why gateway sits at .254:

Placing the gateway at the highest usable address (.254) keeps the low end of the range (.1 through .19) available for future static device assignments, while DHCP pools start at .20 for dynamic clients.

Why 203.0.113.0/24 for public addresses:

This is IANA's documentation-reserved range, guaranteed never to be assigned to a real organization. Using it prevents any risk of overlapping with real public addresses in the simulation.

Why Guest VLAN 99:

Guest devices need internet access but must never reach internal departments or servers. VLAN 99 provides complete Layer 2 and Layer 3 separation from all other VLANs, with GUEST-ACL enforcing isolation at Leaf-1's SVI.

Why a separate VLAN was necessary:

Allowing guests onto any existing department VLAN would place untrusted devices on the same Layer 2 segment as internal workstations, giving them potential access to department resources even if ACLs partially restrict traffic. A dedicated VLAN 99 ensures complete Layer 2 and Layer 3 separation from all other VLANs from the moment a guest device connects.

What guests can access:

- Internet (via NAT through the firewall) ✅

- DMZ-Web (company public website) ✅

What guests cannot access:

- Any internal department (VLANs 10-60) ❌

- Any internal server (VLAN 70 — DNS, DHCP, AAA, Compute, FTP, Syslog) ❌

- iSCSI storage (VLAN 90) ❌

- DMZ-FTP (company file server — restricted to prevent unauthorized uploads) ❌

How isolation is enforced:

GUEST-ACL is applied inbound on Leaf-1's VLAN 99 SVI, denying traffic from 192.168.99.0/24 toward all internal subnets. This ACL was confirmed actively enforcing in Packet Tracer — unlike the department ACLs on VLANs 10-60, which are a documented PT simulation gap.

Physical implementation:

An Access Point (AP-PT) was placed in the Lobby area, connected to Leaf-1 Gig1/0/9 on VLAN 99, broadcasting SSID NVIDIA-Guest with WPA2-PSK authentication. A wired guest laptop is also connected to Leaf-1 Gig1/0/10 on VLAN 99 for demonstration purposes. In production, all guest devices would connect wirelessly through the Access Point — the wired laptop is purely a test device for the Packet Tracer simulation.

DHCP for guests:

A dedicated Guest DHCP pool was created on the DHCP server (192.168.70.11) assigning addresses in the 192.168.99.20-253 range. The ip helper-address on Leaf-1's VLAN 99 SVI forwards DHCP requests to the server, exactly as with all department VLANs.

Why Leaf-1 was chosen for Guest VLAN:

Guest VLAN 99 was placed on Leaf-1 (the user aggregation switch) rather than Leaf-2 (servers) or Leaf-3 (edge) for three reasons:

- All user-facing VLANs already reside on Leaf-1 — consistency

- All ACL enforcement points are on Leaf-1 — single policy management location

- Guest traffic never needs to touch server or edge infrastructure before being filtered