PACKET TRACER LIMITATIONS

### 1. SVI ACL Enforcement (Catalyst 3650)

What we wanted to do: Apply department-level ACLs to each department's VLAN interface (SVI) on Leaf-1, blocking lateral movement between departments while allowing access to shared servers, DMZ, and internet.

What happened: The ACLs were created correctly and verified with show access-lists — every line was present and correct. However, show ip interface vlan X consistently showed Inbound access list is not set even after running ip access-group X-ACL in multiple times, in both named and numbered ACL formats.

The limitation: Packet Tracer's Catalyst 3650 simulation does not reliably enforce ip access-group attachment on SVI interfaces. The command is accepted without error but has no effect.

Production behaviour: On real Catalyst 3650 hardware, this configuration is standard and fully supported — department isolation would be enforced exactly as designed.

### 2. DMZ Isolation ACL Enforcement (Catalyst 3650)

What we wanted to do: Apply DMZ-ISOLATION ACL to Leaf-3's VLAN 80 SVI, preventing DMZ servers from initiating connections to any internal VLAN.

What happened: Same as above — the ACL was created correctly with all 8 deny lines and a permit fallback, but PT did not enforce it once attached to the SVI.

The limitation: Same Catalyst 3650 SVI ACL enforcement gap as Section 1.

Production behaviour: Would fully enforce on real hardware.

### 3. GUEST-ACL Enforcement (Exception — Worked)

What we wanted to do: Apply GUEST-ACL to Leaf-1's VLAN 99 SVI, restricting guest devices to internet and DMZ-Web only.

What happened: Unlike Sections 1 and 2, this ACL was confirmed actively enforcing — guest devices received "Destination host unreachable" when attempting to reach internal departments and servers, while internet and DMZ-Web remained accessible.

The limitation: None — this is noted as a positive exception. The inconsistency between this ACL working and the others not working highlights that PT's SVI ACL enforcement is unpredictable rather than uniformly broken.

### 4. RADIUS Server Configuration (Catalyst 3650)

What we wanted to do: Configure radius-server host 192.168.70.12 auth-port 1645 key Cisco123 on Leaf-1 to point the switch's AAA authentication toward the RADIUS server.

What happened: The command was rejected with % Invalid input detected. Both legacy and modern (radius server block) syntax were attempted — neither persisted in the running configuration.

The limitation: Packet Tracer's Catalyst 3650 simulation does not accept RADIUS server addressing commands.

Production behaviour: Fully supported on real hardware — this is one of the most common AAA configurations in enterprise networking.

### 5. 802.1X Port Control (Catalyst 2960)

What we wanted to do: Apply dot1x port-control auto to IT-Access-Switch ports Fa0/1-5, requiring devices to authenticate via 802.1X before gaining network access.

What happened: The command was accepted without error, but did not appear in the saved running configuration when checked afterward.

The limitation: Packet Tracer's Catalyst 2960 simulation does not persist port-level 802.1X configuration.

Production behaviour: Fully functional on real hardware.

### 6. Syslog on Catalyst Switches

What we wanted to do: Configure logging 192.168.70.16 and logging trap 6 on all Layer 3 switches (Spines, Leaves) to forward log messages to the central Syslog server.

What happened: The logging command was rejected as invalid input on Catalyst 3650 and 2960 simulations.

The limitation: Packet Tracer's switch simulation does not support the logging command.

Production behaviour: Fully supported — this is standard practice for centralized log management.

### 7. Syslog on ASA Firewall

What we wanted to do: Configure logging enable, logging host inside 192.168.70.16, and logging trap 6 on the ASA Firewall.

What happened: All three commands were rejected as invalid input, even when entered correctly inside configure terminal.

The limitation: Packet Tracer's ASA 9.6 simulation does not support logging configuration commands.

Production behaviour: Fully supported on real ASA hardware.

### 8. iSCSI Protocol Simulation

What we wanted to do: Configure the iSCSI Storage server with a functioning iSCSI target service, accessible only to Production department.

What happened: No iSCSI service option exists anywhere in Packet Tracer's Services tab for any server device.

The limitation: Packet Tracer does not simulate the iSCSI protocol at all.

Production behaviour: The server is represented by its IP address and network-level reachability/restriction only; the actual storage protocol cannot be demonstrated in this simulator.

### 9. AI/LLM Workload Simulation

What we wanted to do: Demonstrate Central-Compute-LLM as an active AI compute workload responding to requests from all 6 departments.

What happened: No AI, machine learning, or generic compute service exists in Packet Tracer's Services tab.

The limitation: Packet Tracer has no simulation capability for compute workloads of this kind.

Production behaviour: Represented by network reachability only — every department can reach 192.168.70.14, fulfilling the hybrid model's connectivity requirement even though the workload itself cannot be simulated.

### 10. SFTP Service

What we wanted to do: Use SFTP (SSH File Transfer Protocol, port 22) instead of standard FTP for DMZ-FTP, to provide encrypted file transfer.

What happened: Checked the full Services tab list on a server device — confirmed available services are HTTP, DHCP, DHCPv6, TFTP, DNS, SYSLOG, AAA, NTP, EMAIL, FTP, IoT, VM Management, Radius EAP, PRP. SFTP is not among them.

The limitation: Packet Tracer does not offer an SFTP service.

Production behaviour: SFTP would replace FTP for DMZ-FTP, consolidating to a single encrypted port (22) instead of two unencrypted ports (20, 21).

### 11. Dynamic ARP Inspection (DAI) and DHCP Snooping

What we wanted to do: Configure DHCP Snooping and Dynamic ARP Inspection on all 6 access switches to prevent ARP spoofing attacks within each department VLAN.

What happened: DHCP Snooping commands were accepted, but the binding database that DAI depends on was not consistently created or populated by the simulation.

The limitation: DAI has a hard dependency on a populated DHCP Snooping binding table. Without it, DAI cannot function — and Packet Tracer does not reliably populate this table.

Production behaviour: Both features are standard and fully functional on real Cisco hardware, providing protection against Man-in-the-Middle ARP spoofing attacks.

### 12. StackWise Stacking

What we wanted to do: N/A — not directly attempted, but identified as a production redundancy enhancement for the Leaf switches.

What happened: N/A

The limitation: Packet Tracer does not simulate StackWise switch stacking technology at all.

Production behaviour: Each Leaf would be deployed as a stacked pair of physical switches for hardware-level redundancy beyond what a single device provides.

### 13. Three-Interface ASA Firewall (DMZ Interface)

What we wanted to do: Add a third named interface (Gig1/3, nameif dmz, security-level 50) to the ASA Firewall to force all DMZ traffic — both from internet and from internal departments — through firewall inspection, addressing the architectural gap where internal-to-DMZ traffic bypassed the firewall entirely.

What happened: The configuration commands were accepted and correctly saved in the running-config (show running-config interface gigabitEthernet1/3 confirmed nameif dmz was present). However, show interface gigabitEthernet1/3 continued to report "Available but not configured via nameif," and the firewall itself could not ping DMZ servers connected to this interface — ping 192.168.80.10 and ping 192.168.80.11 from the firewall both failed at 0% success despite physically correct cabling (verified via DMZ-Switch port status showing connected).

The limitation: Packet Tracer's ASA 9.6 simulation does not properly route traffic through a third named interface, even when the interface configuration is syntactically correct and saved.

Production behaviour: A three-interface ASA firewall with inside/outside/DMZ zones is a standard, widely-deployed enterprise pattern, fully functional on real hardware. This would have provided stateful inspection of all DMZ traffic regardless of source.

Resolution: Reverted to the original two-interface design (outside/inside only) with DMZ-ISOLATION ACL on Leaf-3's VLAN 80 SVI, consistent with the RFQ requirement of "DMZ via VLANs and ACLs." This is documented as the primary production enhancement recommendation — see panel talking points.