### Why Leaf-Spine Was Chosen

#### 2.1 Fixed, Predictable Hop Count

In a leaf-spine topology, any device on any Leaf can reach any device on any other Leaf in exactly two hops — one hop up to a Spine, one hop down to the destination Leaf. This is true regardless of which department the source is in or which server the destination is. Every workstation in every department has exactly the same path length to Central-Compute-LLM.

This directly addresses NVIDIA's non-negotiable hybrid model requirement. With 48 workstations all communicating with one centralized server, a fixed two-hop path ensures consistent, predictable latency for every user regardless of their department.

#### 2.2 Native Redundancy via ECMP

With two Spines, every Leaf has two equal-cost paths to every other Leaf. OSPF's Equal-Cost Multi-Path (ECMP) routing automatically load-balances traffic across both paths simultaneously. If one Spine fails, OSPF reconverges and all traffic shifts to the remaining Spine without any manual intervention or configuration change. This redundancy is built into the topology itself — it doesn't require additional protocols like HSRP or VRRP.

#### 2.3 Traffic-Function Leaf Assignment

The most distinctive architectural decision was assigning Leaves by traffic function rather than physical location or department. This decision came directly from the hybrid model requirement.

In this network, every device either asks questions (workstations) or answers them (servers), with one boundary to the outside world. Grouping by role in the request/response cycle produces a cleaner, more logical design than grouping by floor or department:

- Leaf-1 (Compute) — every device that asks. All 48 workstations across all 6 departments connect here via their own dedicated access switches.

- Leaf-2 (Services) — every device that answers. All 7 internal servers live here.

- Leaf-3 (Edge) — the only boundary to the outside world. Firewall, Edge Router, and DMZ servers.

#### 2.4 Scalability

Adding a new department requires only adding a new access switch and connecting it to Leaf-1 — no changes to the Spine layer or other Leaves. Adding a new server requires only connecting it to Leaf-2 — no changes elsewhere. This horizontal scalability makes the design future-proof for the R&D hub's potential growth.

#### 2.5 Why HSRP Was Rejected

HSRP (Hot Standby Router Protocol) pairs two devices doing identical jobs with one active and one standby. In this design, there is only one Leaf-1, one Leaf-2, and one Leaf-3 — no duplicate devices per role. HSRP would add configuration complexity without solving a real problem. Redundancy comes from OSPF ECMP across two Spines, which is the correct mechanism for this topology.

### 3. Final Topology Description

#### 3.1 Physical Components

| Device | Model | Count | Layer | Role |
| --- | --- | --- | --- | --- |
| Spine switches | Catalyst 3650 | 2 | Layer 3 | Core fabric, OSPF routing |
| Leaf switches | Catalyst 3650 | 3 | Layer 3 | Aggregation, SVIs, inter-VLAN routing |
| Access switches | Catalyst 2960 | 6 | Layer 2 | End-device connectivity |
| ASA Firewall | ASA 5506 | 1 | Layer 3/7 | Perimeter security, NAT |
| Edge Router | ISR | 1 | Layer 3 | Internet connectivity |

Note: Catalyst 3650 and 2960 were used because they are the highest-capability models available in Packet Tracer. In a production deployment, Catalyst 9300/9200-class hardware would be used.

#### 3.3 Spine Layer

The two Spines (Spine-1 and Spine-2) form the core of the fabric. They have no VLANs, no SVIs, and no end devices connected to them. Every port on both Spines is a routed Layer 3 point-to-point link facing a Leaf switch. The Spines do not know about department VLANs or server subnets — they only know about the point-to-point subnets connecting them to each Leaf, plus whatever routes OSPF tells them about.

#### 3.4 Leaf Layer

Each Leaf switch serves a specific traffic function:

Leaf-1 (Compute):
 Has six trunk ports connecting to the six department access switches (one per department), one port connecting to the Guest Laptop on VLAN 99, and two routed ports connecting to both Spines. Hosts SVIs for VLANs 10, 20, 30, 40, 50, 60, and 99 — acting as the default gateway for every department and the guest network. Runs OSPF on both Spine-facing interfaces only (passive-interface on all VLAN-facing interfaces).

Leaf-2 (Services):
 Has seven access ports connecting to internal servers (DNS, DHCP, AAA, Central-Compute, Internal-FTP, Syslog on VLAN 70, and iSCSI on VLAN 90), and two routed ports connecting to both Spines. Hosts SVIs for VLANs 70 and 90 — acting as the default gateway for all internal servers. Runs OSPF on both Spine-facing interfaces only.

Leaf-3 (Edge):
 Has one routed port connecting to each Spine, one routed port connecting to the Firewall's inside interface, and two access ports connecting to DMZ-Web and DMZ-FTP on VLAN 80. Hosts SVI for VLAN 80 — acting as the default gateway for DMZ servers. Runs OSPF on Spine-facing interfaces and originates the default route toward the rest of the fabric using default-information originate.

#### 3.5 Access Layer

Six Catalyst 2960 access switches, one per department. Each switch:

- Carries only its own department's single VLAN

- Has a Gigabit trunk uplink to Leaf-1 (restricted to that one VLAN)

- Has FastEthernet access ports for workstations and printers (where applicable)

- Runs Spanning Tree Protocol (PVST) with PortFast enabled on all end-device ports

### 4. How the Topology Works

#### 4.1 Normal Traffic Flow — Workstation to Server

When an IT department PC requests data from Central-Compute-LLM:

- PC → IT-Access-Switch — PC sends frame on VLAN 10. Switch forwards it up the trunk to Leaf-1.

- IT-Access-Switch → Leaf-1 — frame arrives at Leaf-1's trunk port on VLAN 10. Leaf-1's VLAN 10 SVI (192.168.10.254) receives the packet and routes it.

- Leaf-1 → Spine — Leaf-1 looks up 192.168.70.14 (Central-Compute) in its routing table. OSPF has told it to reach 192.168.70.0/24 via both Spines. ECMP selects one Spine — packet goes out Gig1/0/1 or Gig1/0/2.

- Spine → Leaf-2 — Spine receives the packet and looks up 192.168.70.0/24. Its routing table says go to Leaf-2. Packet forwarded to Leaf-2.

- Leaf-2 → Central-Compute — Leaf-2 receives packet, looks up 192.168.70.14. It's directly connected on VLAN 70. Leaf-2 ARPs for the server's MAC address and forwards the frame to the server's port.

- Reply follows the reverse path — Central-Compute replies to 192.168.10.x. Same path in reverse: Leaf-2 → Spine → Leaf-1 → IT-Access-Switch → PC.

Total hops from PC to server: 5 devices (Access Switch → Leaf-1 → Spine → Leaf-2 → Server). Fixed and consistent for every department.

#### 4.2 Normal Traffic Flow — Internet Access

When a Production PC browses the internet:

- PC sends packet to its gateway (192.168.20.254 on Leaf-1).

- Leaf-1 looks up destination — not in any known subnet, matches default route (O*E2 learned from Leaf-3 via OSPF).

- Packet travels Leaf-1 → Spine → Leaf-3 → Firewall inside interface.

- Firewall performs NAT/PAT — translates source 192.168.20.x to 203.0.113.6 (firewall's outside IP).

- Packet exits firewall outside interface → Edge Router → Internet Cloud.

- Reply arrives at Edge Router → Firewall outside → NAT untranslation → Firewall inside → Leaf-3 → Spine → Leaf-1 → PC.

#### 4.3 Normal Traffic Flow — Internet to DMZ

When an external visitor accesses the company website:

- Request arrives at Edge Router from Internet Cloud.

- Edge Router forwards to Firewall outside interface (203.0.113.6).

- Firewall checks OUTSIDE-IN ACL — TCP port 80 to 192.168.80.10 is permitted.

- Firewall routes packet to inside interface → Leaf-3 → VLAN 80 → DMZ-Web.

- DMZ-Web sends reply → Leaf-3 → Firewall inside → Firewall outside → Edge Router → Internet.

#### 4.4 Redundancy — Spine Failure

If Spine-1 fails:

- OSPF detects loss of adjacency with Spine-1 on all connected Leaves.

- OSPF removes all routes learned via Spine-1 from routing tables.

- All traffic automatically shifts to Spine-2 — no configuration change, no manual intervention.

- Network continues operating normally with slightly reduced bandwidth capacity.

- When Spine-1 recovers, OSPF re-establishes adjacency and ECMP load-balancing resumes automatically.

#### 4.5 DHCP Process

When a new PC connects to the network:

- PC sends DHCP Discover broadcast on its VLAN.

- Broadcast reaches Leaf-1's SVI for that VLAN.

- Leaf-1's ip helper-address forwards the broadcast as a unicast to DHCP server (192.168.70.11).

- DHCP server checks which pool matches the request (identified by the relay agent's source IP — the SVI address).

- DHCP server sends Offer back via Leaf-1 to the PC.

- PC accepts the offer, receives IP address, subnet mask, gateway, and DNS server.

#### 4.6 Routing Protocol — OSPF

OSPF Area 0 runs across all 5 Layer 3 switches. Key design decisions:

passive-interface default: OSPF hello packets are only sent on Spine-Leaf point-to-point links — never toward end devices (PCs, servers, printers). This is configured using passive-interface default with explicit no passive-interface on Spine-facing ports only.

Default route propagation: Leaf-3 has a static default route pointing to the firewall (10.0.3.2). The default-information originate command causes OSPF to advertise this default route to all other OSPF speakers. Leaf-1 and Leaf-2 receive the default route automatically as O*E2 without any manual configuration.

ECMP: Both Spine paths have identical OSPF cost. OSPF installs both paths in the routing table simultaneously and load-balances traffic across both — verified by checking routing tables which show two equal-cost next-hops for every remote subnet.

### 5. Simulation vs Production Differences

| Aspect | Packet Tracer Simulation | Production Deployment |
| --- | --- | --- |
| Hardware | Catalyst 3650/2960 | Catalyst 9300/9200 |
| Leaf redundancy | Single Leaf per role | StackWise-stacked Leaf pairs |
| SVI ACL enforcement | Not enforced (PT limitation) | Fully enforced |
| RADIUS authentication | Not fully simulated | Fully functional |
| iSCSI protocol | Not simulated | Full block storage |
| Internet connectivity | Simulated via loopback | Real internet routing |
| Syslog on switches | Not supported in PT | Fully functional |